# The Maker’s Journey:	 Measurement Mission

Pulley Power

Build a model pulley and line shaft system to investigate power transmission and gear ratios.

Materials:

Cardboard Wheels (3 sizes)

Chipboard Wheels

Paper Straws

Glue

Paper cup

Paint brush

Rubber band

Cardboard arrow template

Binder clip

Paper ruler

Wheel Circumference Measurements. Use the paper ruler to measure the circumference of each of the 3 different sized cardboard wheels to the nearest inch.

| Wheel Size | Circumference (inches) |
| --- | --- |
| Small |  |
| Medium |  |
| Large |  |

Construct Line Shaft Pulleys.

- On one straw, glue on 3 medium wheels and 2 chipboard circles – one on each side to hold the rubber band in place.

- On the other straw glue on 3 wheels of each of the 3 sizes in order from smallest to largest. Allow both straws time to dry.

GLUEING TIPS:

- Apply the glue with a paintbrush to get a thin layer over the entire surface of the wheel. This will allow for quick drying time.

- Also apply some glue to the straw so the wheels attach to the straw and won’t slip.

- Cut the Cardboard Arrow template in half and trim the ends without the hole into points (arrows). Place one arrow onto each straw BUT DO NOT GLUE IT ON. It should fit snuggly, but it is important that you are able to spin it freely for the experiment.

Pulley Power

For this experiment, the input axle will always be the one with the 3 medium sized wheels and chipboard circles. The output axle is the one with 3 different sized wheels.

- Write your measured circumferences for each wheel on the table below. The Input Wheel Size column will all be the same – the medium sized wheel. In the Output Wheel Size column, write the sizes starting with the smallest in the first open row, medium in the second, and largest in the third.

- Loop the rubber band around the wheels on both straws axles. One partner will hold the input axle and the other the output.

- Start with the rubber band around the input wheels and the smallest wheel on the other.

- Practice spinning to make sure you have the rubber band at the correct tightness to easily spin the axles.

- When ready for the first test, adjust the cardboard arrows so they are both pointing in the same direction (up).

- The holder of the input axle will gently spin their axle 10 times around. Use the arrow on the input axle to count the 10 rotations.

- The holder of the output axle counts the number of rotations their axle makes.

- Record below.

| Pulley Combination | Input Wheel Size (in inches) | Output Wheel Size (in inches) | # of Input Rotations | # of Output Rotations | Speed Ratio (Output/Input) |
| --- | --- | --- | --- | --- | --- |
| Combo 1 |  |  | 10 |  |  |
| Combo 2 |  |  | 10 |  |  |
| Combo 3 |  |  | 10 |  |  |

Reflection Questions

- Which pulley combination produced the fastest output rotation? Why?

- How would changing the input pulley size affect the output speed?

- How might this system have helped factories operate more efficiently?

- Think about how this relates to the gears on a bike. Where do you want the bigger gear (front or back) to go fastest?